### Write a program to compute CRC code for the polynomials CRC-12, CRC-16 and CRC CCIP

Cyclic Redundancy Check

This Cyclic Redundancy Check is the most powerful and easy to implement technique. Unlike checksum scheme, which is based on addition, CRC is based on binary division. In CRC, a sequence of redundant bits, called cyclic redundancy check bits, are appended to the end of data unit so that the resulting data unit becomes exactly divisible by a second, predetermined binary number. At the destination, the incoming data unit is divided by the same number. If at this step there is no remainder, the data unit is assumed to be correct and is therefore accepted.

A remainder indicates that the data unit has been damaged in transit and therefore must be rejected.

  - Bit strings are created as representation of polynomials with coefficients ‘0’ and ‘1’only.

  - A k-bit frame is regarded as coefficients list for a polynomial with ‘k’ terms (xk-1 to x0 ) Eg: x5 + x4 +x0 = 110001

When this method is used, the sender and the receiver should agree upon a generator polynomial, G(x) in advance.

Both the high and low order bits of G(x) must be ‘1’

To compute checksum for some frame with ‘m’ bits (polynomial = M(x), append ‘r’ zero bits to the lower end of the frame (r = degree of the generator polynomial) so that this check summed frame is divisible by G(x).

Divide M(x) by G(x) using modulo-2 division and subtract the remainder from M(x) using modulo-2 subtraction. let the resultant be called as T(x)

T(x) is passed to the receiver and the receiver divides it by G(x). If there is a remainder, there has been a transmission error.

Eg: frame = 1101011011 G(x) = x4 +x +1 = 10011

è degree = 4

Therefore, frame = 1101011011 + 0000

èM(x) = 11010110110000

Commonly used divisor polynomials are: CRC 12: x12 + x11 + x3 + x2 + x + 1

CRC 16: x16 + x15 + x2 + 1

CRC CCITT: x16 + x12 + x5 + 1

Program:-

import java.util.Scanner;

public class CRC

{

static String data, divisor;

static char[] temp, total;

static int dataLen, divLen, len;

static String check(char[] totalData) {

char[] dataArray = new char[divLen];

for (int i = 0; i < divLen; i++)

dataArray[i] = totalData[i];

int j = divLen;

while (j <= len) {

if (dataArray[0] == '1') {

for (int i = 1; i < divLen; i++) {

if (dataArray[i] == divisor.charAt(i))

dataArray[i] = '0';

else

dataArray[i] = '1';

}

}

for (int i = 0; i < divLen - 1; i++)

dataArray[i] = dataArray[i + 1];

if (j < totalData.length)

dataArray[divLen - 1] = totalData[j];

j++;

}

return new String(dataArray, 0, divLen - 1);

}

public static void main(String[] args) {

Scanner sc = new Scanner(System.in);

System.out.print("Enter the total bit of data: ");

dataLen = sc.nextInt();

System.out.print("Enter 0 for CRC-12 and 1 for CRC-16: ");

int choice = sc.nextInt();

if (choice == 0) {

divisor = "1100000001111";        // CRC-12

divLen = 13;

}

else {

divisor = "11000000000000101";    // CRC-16

divLen = 17;

}

len = dataLen + divLen - 1;

System.out.print("Enter the data: ");

data = sc.next();

total = new char[len];

temp = new char[len];

for (int i = 0; i < dataLen; i++) {

total[i] = data.charAt(i);

temp[i] = data.charAt(i);

}

for (int i = dataLen; i < len; i++)

total[i] = '0';

String remainder = check(total);

for (int i = 0; i < divLen - 1; i++)

temp[dataLen + i] = remainder.charAt(i);

System.out.println("\nRemainder = " + remainder);

System.out.println("Divisor = " + divisor);

System.out.println("Transmitted Code Word = " + new String(temp));

System.out.print("\nEnter the received code word: ");

String received = sc.next();

total = received.toCharArray();

remainder = check(total);

boolean error = false;

for (int i = 0; i < divLen - 1; i++) {

if (remainder.charAt(i) == '1') {

error = true;

break;

}

}

if (error)

System.out.println("Received code word contains errors...!!!");

else

System.out.println("Received code word contains no errors...!!!");

sc.close();

}

}